Carrier Aggregation & OFDM Throughput (LTE → LTE-Advanced)

Objective:

- Simulate OFDM transmission over 1-carrier vs 2-carrier vs 3-carrier aggregation.

- Measure throughput under different SNR and interference levels.

Theory:

### 1. OFDM Principle

OFDM divides the channel into many orthogonal subcarriers, making transmission resistant to multipath fading and allowing efficient use of bandwidth.

### 2. Carrier Aggregation (CA)

LTE-Advanced combines multiple component carriers (e.g., 1CC, 2CC, 3CC) to form a wider effective bandwidth, increasing total user throughput.

### 3. Impact of SNR

Higher SNR enables higher-order modulation (QPSK → 16-QAM → 64-QAM), increasing bits per symbol and improving throughput.

### 4. Frequency-Selective Fading

Each carrier experiences different fading conditions; CA provides frequency diversity so weak carriers are compensated by stronger ones.

### 5. Effect of Interference

Interference reduces throughput on individual carriers, but aggregated carriers maintain higher overall throughput compared to single-carrier LTE.

Procedure:

- Set system parameters: number of carriers (1CC / 2CC / 3CC), subcarriers per carrier, modulation order, and SNR.

- Generate random data bits for the user based on the total available bandwidth.

- Map bits to QAM symbols (QPSK / 16-QAM / 64-QAM depending on SNR).

- Form OFDM symbols by assigning QAM symbols to subcarriers and applying IFFT for each carrier.

- Add cyclic prefix (CP) to model realistic LTE/LTE-A transmission.

- Apply Carrier Aggregation by combining the OFDM signals from all component carriers.

- Pass the signal through channel models, including noise and optional interference.

- Calculate per-carrier throughput and total aggregated throughput.

- Plot comparison graphs for 1-carrier vs 2-carrier vs 3-carrier:

- Throughput vs SNR

- Interference effect vs throughput

- Display key results: bits per symbol, throughput per carrier, and overall CA throughput.

Code:

clear; clc; close all;

%% ----------------------------------

% Parameters

%% ----------------------------------

BW = 20e6;                 % Bandwidth per carrier (20 MHz)

numSubcarriers = 1024;     % OFDM size

SNR_dB = 0:5:30;           % SNR range

SNR = 10.^(SNR_dB/10);

numIter = 200;             % Monte-Carlo iterations

% Carrier Aggregation counts

CA_list = [1, 2, 3];

% Frequency-selective fading variance for each carrier

fadingVar = [1.0, 0.7, 0.5];

% Interference factor (0 = no interference, 0.3 = 30% interference)

interferenceFactor = 0.3;

% Storage

throughput = zeros(length(CA_list), length(SNR));

%% ----------------------------------

% Simulation

%% ----------------------------------

for c = 1:length(CA_list)

numCarriers = CA_list(c);

for s = 1:length(SNR)

rate = 0;

for n = 1:numIter

totalCapacity = 0;

for k = 1:numCarriers

% Channel gain for OFDM (frequency selective)

h = sqrt(fadingVar(k)/2) * (randn(1, numSubcarriers) + 1i*randn(1, numSubcarriers));

% Interference power

interference = interferenceFactor * SNR(s);

% SINR for each subcarrier

SINR_sub = (SNR(s) * abs(h).^2) ./ (1 + interference);

% Shannon capacity for each subcarrier

totalCapacity = totalCapacity + sum(log2(1 + SINR_sub));

end

rate = rate + totalCapacity;

end

% Convert to Mbps

throughput(c, s) = (BW * (rate/numIter)) / 1e6;

end

end

%% ----------------------------------

% Plotting

%% ----------------------------------

figure;

plot(SNR_dB, throughput(1,:), 'k-o', 'LineWidth', 1.8); hold on;

plot(SNR_dB, throughput(2,:), 'r-s', 'LineWidth', 1.8);

plot(SNR_dB, throughput(3,:), 'b-^', 'LineWidth', 1.8);

grid on;

xlabel('SNR (dB)');

ylabel('Throughput (Mbps)');

title('Carrier Aggregation & OFDM Throughput (LTE → LTE-Advanced)');

legend('1-Carrier (LTE)', '2-Carrier CA', '3-Carrier CA', 'Location', 'NorthWest');

%% ----------------------------------

% Console Output at SNR = 20 dB

%% ----------------------------------

target = find(SNR_dB == 20);

fprintf("\n--- Throughput Summary at 20 dB ---\n");

fprintf("1-Carrier LTE:         %.2f Mbps\n", throughput(1, target));

fprintf("2-Carrier LTE-A:       %.2f Mbps\n", throughput(2, target));

fprintf("3-Carrier LTE-A Pro:   %.2f Mbps\n", throughput(3, target));

fprintf("\nObservations:\n");

fprintf("1. Throughput increases almost linearly with number of aggregated carriers.\n");

fprintf("2. Frequency-selective fading (1.0, 0.7, 0.5) reduces extra gain from each carrier.\n");

fprintf("3. Interference reduces SINR → flattening slopes.\n");

fprintf("4. OFDM allows independent SINR per subcarrier → realistic CA behavior.\n");

OUTPUT:

CONSOLE OUTPUT:

Throughput Summary at 20 dB ---
1-Carrier LTE:         35562.24 Mbps
2-Carrier LTE-A:       64735.80 Mbps
3-Carrier LTE-A Pro:   88702.31 Mbps

Conclusion:

1. Throughput increases almost linearly with number of aggregated carriers.

2. Frequency-selective fading (1.0, 0.7, 0.5) reduces extra gain from each carrier.

3. Interference reduces SINR → flattening slopes.

4. OFDM allows independent SINR per subcarrier → realistic CA behavior.